Project Overview

This project automates plant watering based on environmental conditions. It helps conserve water and ensures optimal soil conditions by using real-time sensor data and smart control logic.

When soil moisture drops below a defined threshold, the system automatically turns ON the water pump. Once optimal moisture is reached, the pump is turned OFF.

Features

- Temperature & Humidity monitoring using DHT11 sensor

- Soil moisture level detection

- Relay-based pump control

- Real-time dashboard using Node-RED

- IoT-based monitoring and automation

- Automatic and manual pump control modes

- Remote access via browser dashboard

Hardware Requirements

- NodeMCU (ESP8266) / ESP32

- DHT11 Temperature & Humidity Sensor

- Soil Moisture Sensor

- 1-Channel Relay Module

- Water Pump (DC/AC depending on setup)

- Power Supply

- Jumper wires + Breadboard

Software Requirements

- Node-RED

- Arduino IDE / PlatformIO

- MQTT Broker (e.g., Mosquitto)

- ESP board libraries

- Node-RED Dashboard nodes

System Architecture

- Sensors collect data:

  - Soil moisture

  - Temperature

  - Humidity

- Microcontroller processes readings.

- Data is sent via MQTT / HTTP to Node-RED.

- Node-RED:

  - Displays dashboard

  - Applies control logic

  - Sends relay ON/OFF commands

- Relay controls water pump.

Node-RED Dashboard

The dashboard includes:

- Real-time soil moisture

- Temperature & humidity charts

- Pump ON/OFF indicator

Working Logic

- If soil moisture < threshold → Pump ON

- If soil moisture ≥ threshold → Pump OFF

- Manual override can bypass automation

- Continuous sensor monitoring every few seconds